Working Principle

The NodeMCU continuously reads data from the connected sensors.

Depending on the sensor values and user inputs, it automatically controls LEDs, the buzzer, and the security functions.

The complete project consists of five integrated modules.

Module 1 – Temperature-Based Fan Control

First, I will demonstrate the temperature-based fan control.

The NTC temperature sensor continuously monitors the surrounding temperature.

Whenever the temperature crosses the preset threshold, the 1st Red LED, which represents the fan, automatically turns ON.

At the same time, the buzzer sounds for two seconds to indicate high temperature.

When the temperature returns to normal, the fan automatically turns OFF.

Module 2 – Automatic Lighting System

Next is the automatic lighting system.

This module is designed to automatically control the house light using an LDR sensor.

However, the hardware kit provided to me is missing the actual LDR sensing element (photoresistor) on the sensor module.

Because of this, the module cannot detect changes in ambient light, and the house light LED remains ON continuously.

The software logic has been implemented correctly and was verified during simulation, but due to the incomplete hardware, live light detection cannot be demonstrated.

Module 3 – Intruder Alert System

Next is the intruder alert system.

The IR sensor continuously detects human motion.

When Security Mode is OFF and motion is detected, the red LED turns ON and the buzzer gives a single beep.

This immediately alerts the user about the presence of a person without activating the full security alarm.

Module 4 – Smart Door Control

Now I will demonstrate the smart door control module.

When the doorbell push button is pressed, the welcome LED turns ON, indicating that the door has been activated.

Originally, this module was designed to operate using a servo motor, which rotates to a specific angle to open and close the door automatically.

However, the hardware kit provided only included a two-wire DC motor and did not include either a servo motor or a motor driver such as the L293D.

A NodeMCU cannot safely control a two-wire DC motor directly because it requires higher current and a dedicated driver circuit.

Therefore, the complete software logic for the smart door has been implemented successfully, but the physical door movement could not be demonstrated due to the missing hardware.

Module 5 – Security Mode

Finally, I will demonstrate the security mode.

When the Security Mode button is pressed, the system enters security mode.

Now, whenever motion is detected by the PIR sensor,

the red LED continuously flashes,

the buzzer continuously sounds,

and the Serial Monitor displays "Motion Detected!"

This indicates that an intrusion has been detected.

When the Security Mode is turned OFF, the system returns to its normal monitoring mode.

Challenges Faced

During this project, I faced several practical challenges but with me and my friend worked alongside and supported each other to overcome the challenges and it took almost 18 hours to build it.

Final Result

The final project successfully integrates multiple smart home automation features into a single embedded system.

The system can:

- Monitor temperature and control a fan.

- Detect ambient light and control house lighting through the implemented software logic.

- Detect human motion.

- Provide an intruder alert.

- Operate a security mode.

- Implement smart door control logic.

Overall, this project demonstrates the practical application of IoT sensors, embedded systems, and microcontroller programming for smart home automation.

Conclusion

This project has helped me improve my understanding of embedded systems, sensor interfacing, hardware troubleshooting, and embedded programming.

I sincerely thank LaunchED Global for providing me with this valuable learning opportunity and practical exposure to IoT and Robotics.

Thank you for watching my project demonstration.